ACE Weekly Report :5/26/99

Spacecraft Activities:

- All spacecraft subsystems are performing nominally. Successsfully performed an attitude maneuver on 5/13.

- Successfully performed an attitude maneuver on 5/25.

Pre-Maneuver    HGA = _____-3.35_____     Sun Angle =  _____5.9__

Post-Maneuver  HGA = _____4.3_______     Sun Angle =  _____9.14

- Executed a bin dump of the active C&DH Block Bins

Ground System Activities:

- The ISIS protos server crashed prior to AOS on 5/25.  IPNOCCwas contacted and ISIS brought back online at AOS

- On 5/26 -  Big Pipe between Madrid and JPL  down due to problems for entire support.  Telemetry received from Little pipe.  Due to little pipe bandwidth of only 64 kbs vc2 output was disabled from D61 and recorded at site for later playback to CDR assembly at JPL

Instrument Activities:

- On 5/20 executed OCR SWM-015 to load the Flight Table for the SWIM instrument.  On

- The weekly SWEPAM calibrations were successfully performed on 5/21.

Y2K Testing:

- On 5/25 the hard drive on MOR worksatation 3 crashed.  The hardware had to be replaced and the UNIX operating system had to be reinstalled on that workstation.  2 full days of Y2K testing were lost due to this problem.  The Y2K team is working to resolve some remaining problems with the operations of the DPS s/w component in the MOR.  This task must be resolved before the passwords can be changed.  Once the passwords are changed the FOT will verify system operational configuration.  All these steps must be completed before a real-time pass can be conducted from the MOR.  Executing a real-time pass from the string is the final step in the FOT Y2k testing.  The earliest possible date planned for executing a real-time pass from the MOR is now set to Tuesday June 1.