ACE Weekly Report: 01/28/2000

All ACE spacecraft subsystems are performing nominally. A successful attitude maneuver was completed on 01/28/00.  Upcoming attitude maneuvers are scheduled for 02/08/00.

On 01/20/00, a contact was scheduled with the spacecraft in order to test DSN 26 meter automation. The test was not successful due to problems with the processing and routing of the telemetry. The MOC saw 4 partial blocks of telemetry. This invalid data caused a corruption of the clock correlation file. A work around was used to get a valid s/c time on the following pass. The previously saved file from the day before the corruption was copied over the current file and all data products were then successfully transmitted. Another 26 meter automation test pass is scheduled for 01/31/00.

On 01/22/00 and 1/23/00, the ACE IMOC packet extractor crashed. The system was successfully rebooted. All SSR-A data slice that were lost during the crash were recovered. Refer to anomaly report AR G00-067. The crash was later repeated when a UNIX script was executed from the wrong workstation.